THESIS INFORMATION PAGE

PhD thesis title: Application of anaerobic membrane bioreactor combined with algal–bacterial consortium for two-stage co-digestion food waste and canteen wastewater to recover biogas and nutrients

Field of study: Environmental Engineering

Field code: 9.52.03.20.

Full name of PhD student: Le Thanh Son

Academic year: 2021

Scientific instructor: Assoc. Prof. Dr. Bui Xuan Thanh

Assoc. Prof. Dr. Le Duc Trung

Institution: Insitute for Environment and Resources, Vietnam National University Ho Chi Minh City

- ABSTRACT

The co-digestion of organic waste and wastewater has emerged as a viable approach for energy recovery and waste management. Traditional anaerobic processes, however, face challenges with severe organic loading rates (OLRs) during long-term treatment. To address this, a two-stage Anaerobic Membrane Bioreactor (2S-AnMBR) was developed, consisting of a hydrolysis reactor (HR) and an anaerobic membrane bioreactor (AnMBR). Operating at temperatures of 25–32 °C, the system expanded OLRs to optimize biogas yield, nitrogen recovery, and membrane fouling. Specific methane production reached 249 ± 7 L CH4/kgCODremoved at an OLR of 6.9 kg COD/m3.d. The HR achieved 21.6% TCOD loss, with efficient hydrolysis despite a short hydraulic retention time (HRT).

In a parallel study, microbial community analysis of the 2S-AnMBR revealed dominance by Bacteroidetes and Firmicutes in the HR, with Lactobacillus enriched due to food waste fermentation. The UASB exhibited dominance by Methanobacterium and Methanosaeta, suggesting hydrogenotrophic and acetoclastic methanogenesis pathways. Although hydrogen generated in the HR contributed to methane production, limited volatile fatty acids (VFA) and competitive bacteria like Acinetobacter affected Methanosaeta abundance. Despite this, the UASB demonstrated efficient methane production and metabolic diversity.

Transitioning from the 2S-AnMBR system, a study investigated the Revolving Algae Biofilm Reactor (RABR) for nutrient removal and algal biomass production. Using synthetic and real wastewater, the study aimed to optimize nitrogen loading rates (NLRs) for biomass growth and nutrient removal. The first stage with synthetic wastewater achieved peak biomass productivity at an NLR of 0.03 kg N/m3.d, with corresponding chlorophyll-a concentrations indicating enhanced photosynthetic activity. In the second stage, employing real wastewater from 2S-AnMBR system, lower biomass productivity but notable COD removal efficiencies were observed, with NH4+-N removal dynamics varying with NLRs.

- NOVELTY

The combined 2S-AnMBR and RABR system offers significant advantages over traditional wastewater treatment technologies, demonstrating considerable environmental, economic, and social superiority. Environmentally, this advanced system is generally effective in removing pollutants, including organic matter, nutrients, and pathogens, ensuring compliance with stringent wastewater standards such as QCVN 14:2008/BTNMT. The system generates less sludge, reduces greenhouse gas emissions, and consumes less energy compared to conventional aerobic systems. Economically, the system's reduced energy demand, lower sludge treatment costs, easy harvesting of algal biomass, and the potential use of biogas result in significant operational cost savings. The system's compact design further reduces infrastructure and maintenance costs, making it an attractive financial option for both urban and industrial applications. Socially, the system's wastewater quality minimizes pathogen and odor risks to public health, enhancing community acceptance.

- THESIS APPLICATION AND ISSUES FOR FURTHER RESEARCH

  - Applicability in practice

The economic and environmental potential for the comprehensive utilization of waste resources found in waste or the final waste generated after anaerobic digestion (digestate) is significant. One of the promising potential applications is the use of waste as a soil conditioner or biofertilizer due to its stability and rich nutrient content. Regarding the algae cultivation model, the efficiency of the RABR system in removing nitrogen compounds (ammonium and nitrate) from wastewater highlights the system's potential as a biological nutrient removal technology. This application is particularly suitable for municipal wastewater treatment plants seeking sustainable and cost-effective solutions to comply with stringent environmental regulations. Algal biomass can serve as a naturally regenerating medium for nitrogen removal, reducing reliance on chemical treatment methods and enhancing overall treatment efficiency. In agriculture, algal biomass shows promise for use as a biofertilizer. The nutrient-rich composition of the biomass, particularly phosphorus, makes it a valuable supplement for enhancing soil fertility.

  - Futher research

Regarding the 2S-AnMBR model, more detailed research is needed on the impact of specific VFA components such as acetic acid, propionic acid, butyric acid, etc., on the degradation and methanation processes. These studies can help optimize operating conditions to enhance methanation.

For the RABR model, the lack of evaluation of the bacterial community in this study is also a shortcoming, especially concerning ammonia-oxidizing bacteria (AOB) and nitrite-oxidizing bacteria (NOB). Therefore, in the future, changes in the bacterial populations on the biofilm during continuous operation must be thoroughly studied to more comprehensively assess the RABR system. The current attached material used is made of polyester and is thin, which during operation leads to stretching, causing the membrane to slip off the axis, which is not ideal. Replacing the membrane may improve performance.

Scientific instructor				PhD student

Assoc. Prof. Dr. Bui Xuan Thanh                              Le Thanh Son

Assoc. Prof. Dr. Le Duc Trung